Blood Glucose Test

Principle

A blood glucose test measures glucose levels in a person’s blood. It can involve pricking of the finger or a blood draw from your vein. This test is usually used by healthcare providers to screen for Type 2 diabetes. This test is beneficial because it helps for managing diabetes by monitoring it regularly. This test can even provide insights into various metabolic disorders and diseases other than diabetes, like heart disease, that can help manage and modify a patients lifestyle to prevent complications.

Specimen requirements

- Criterias:

- Specimen Collection Criteria: For capillary finger-prick, the puncture site is cleaned with an antiseptic or alcohol swab. After cleaning, the skin needs to dry completely before pricking.  the first drop of blood is wiped off then the subsequent drop is collected in a slide. For venous laboratory draw, a phlebotomist collects a sample of blood from a vein into an appropriate collection tube.

- Labeling: Ensure patient identification is matched with the manual verification or barcode before sample application.

- Rejection Criteria: Samples are usually rejected because of insufficient volume, delays in transport, labeling error, incorrect collection tubes, or if they are taken from an arm currently receiving IVF.

- Storage & Transport: Capillary blood must be tested immediately upon collection; no specimen storage or transport is permitted. For venous blood, it must be processed, cooled, or centrifuged quickly to prevent glycolysis.

- Procedures for submission to central labs:

Submitting blood samples to a central laboratory for blood glucose testing requires strict adherence to collecting, handling, and transport protocols prevent skewing results.

- Sample collection: Venous blood is drawn via venipuncture and collected into a mint green top tube or a glycolytic inhibitor tube. For patients who are ordered by the physician to fast, ensure that they have fasted for atleast 8 to 12 hours.

- Processing and handling: blood samples must be processed by separating the plasma or serum from the cellular fraction within 30 to 45 minutes of collection when using standard tubes. If separation is delayed, maintain samples at room temperature or place them immediately in an ice-water slurry according to specific lab specifications. Additionally, using tubes with rapid glycolysis inhibitors preserves sample stability when prompt separation is not possible.

- Transport: Samples must be correctly labeled with complete patient identifiers and specific collection times, and as much as possible avoid the criterias for rejection.

Quality Control

- Control material

Blood glucose testing uses control solution; level 1 (low range) for low blood sugar levels, level 2 (normal range) for standard target ranges, and level 3 (high range) for elevated glucose concentration.

- Preparation, handling, and storage

Store control solutions unopened at room tempreature or refrigerated (2 degrees - 30 degrees celsius). Do not freeze. Once opened, mark the bottle with the open date and discard date per manufacturer specifications. Mix gently before dispensing; discard the first drop to clear the nozzle before testing.

- Frequency of testing

Frequency of testing is based on the specific type of diabetes and treatment plan.

- Type 1 diabetes (requires frequent monitoring): must check their levels between 4 to 10 times everyday.

- Type 2 diabetes (testing varies): test several times a day or only a few times a week, depends on insulin use.

- Prediabetes or general screening: test HbA1c levels every 3 to 12 months as advised by doctors.

- Expected results

Fasting Blood Glucose Test:

- Normal: 70 to 99 mg/dL (3.9 to 5.5 mmol/L)

- Prediabetes: 100 to 125 mg/dL (5.6 to 6.9 mmol/L)

- Diabetes: 126 mg/dL (7.0 mmol/L) or higher

- Corrective actions

Corrective actions can imply with different aspects in blood glucose testing; like equipment errors, sample collections, and patient value.

Equipment and Error Corrections:

When handling glucose meter errors or unusual readings, removing the test strip, turn off the device, and check the use manual for troubleshooting steps. Always discard expired or damaged test strips, ensuring new strips match the code on the meter vial. If you suspect an inaccurate reading or open a fresh bottle of test strips, perform a control solution test to verify accuracy. Finally, replace the batteries immediately as soon as a low-battery warning appears.

Sample collection Correction:

If you get an inadequate blood drop, avoid squeezing or milking the finger, as this dilutes the blood with tissue fluid; instead, warm your hands or prick a new spot. If you suspect the sample is contaminated with alcohol or food residue, wash your hands thoroughly with soap and warm water, dry them completely, and re-test.

Patient Value Corrections:

- Hypoglycemia (low blood sugar): administer 15 grams of fast-acting carbohydrates and recheck in 15 minutes.

- Hyperglycemia (high blood sugar): follow the prescribed sliding-scale insulin protocol or contact a healthcare provider if levels remain severely elevated.

- Recording and storage of QC data:

When recording for patient data, automatic transmission to the LIS upon docking or manual entry in the POC Quality Assurance Log sheet is needed to safely store those data. Make sure that the QC logs are retained for a minimum of 2 years or per regulatory requirements.

- Alternatives (if no QC materials are available):

Testing a fresh patient sample on a backup meter and immediately re-testing it on a primary laboratory reference analyzer helps verify correlation. Laboratories can also apply patient-based algorithms, such as tracking moving averages across historical data, to identify systematic analytical bias.

Limitations of Methods

- Hematocrit Effect: extreme RBC levels distort results. Severe anemia can cause falsely elevated readings, while polycythemia can cause falsely lowered readings.

- Interfering Substances: Common over-the-counter medications and natural bodily compounds can chemically interfere with the test strip, leading to falsely high glucose results.

- Hypoperfusion and Shock: capillary fingerstick tests become highly inaccurate if a patient has low blood pressure, dehydration, or may be in shock. In these critical state, blood must be drawn from a vein or artery for central lab testing.

- Test Strip Exposure: Environmental factors like heat, light, and humidity degrade the active chemicals on the strips, ruining their reliability.

References

Professional, C. C. M. (2026c, February 16). Blood glucose (Sugar) test. Cleveland Clinic. https://my.clevelandclinic.org/health/diagnostics/12363-blood-glucose-test

5 important reasons for a blood sugar test. (n.d.-b). Primary Care & Family Medicine Located in Omaha and Gretna, Nebraska | Midwest Regional Health Services. https://www.mrhsomaha.com/post/5-important-reasons-for-a-blood-sugar-test

RegisteredNurseRN. (2023b, November 27). How to check blood sugar level (Glucose) | Glucometer diabetes testing procedure nursing [Video]. YouTube. https://www.youtube.com/watch?v=RsGzFs9jfOA

Blood glucose test. (n.d.). https://medlineplus.gov/lab-tests/blood-glucose-test/

Mathew, T. K., & Zubair, M. (2026, April 12). Blood Glucose Monitoring. StatPearls - NCBI Bookshelf. https://www.ncbi.nlm.nih.gov/books/NBK555976/

Control Solution: What it is and how to use it | Diathrive. (n.d.). https://www.diathrive.com/enterprise/resources/post/whats-control-solution

Nwachukwu, J., & Nwachukwu, J. (2026, March 10). How often should you do high blood sugar testing? SC Internal Medicine Assoc & Rehab. https://www.scinternalmedicine.com/2026/03/06/how-often-should-you-do-high-blood-sugar-testing/