COMPLETE BLOOD COUNT (CBC)

QUALITY ASSURANCE

LABORATORY INSTRUCTIONS

By Xavier S. Famorcan

Principle

Complete Blood Count (CBC) is a type of blood test that evaluates the amount of red blood cells, white blood cells, hemoglobin, hematocrit, and platelets in a blood sample. These tests can determine if patients have anemia, erythrocytosis, leukocytosis, leukopenia, thrombocytosis, and thrombocytopenia.

Specimen requirements

Specimen Collection

- Whole blood

- Perform venipuncture to Fill 300 microliters with a 3 mL lavender (EDTA anticoagulant) top tube or Lavender (K2EDTA)

Labeling

- Label Immediately

- Patient's name, date of birth, medical record number, date and time of collection, and test ordered

Rejection,

- Clotted Specimen, insufficient specimen amount, improper labeling, hemolysis, delay in transport.

Storage

- Refrigerated at 2-8°C

Transport

- Transport to the laboratory promptly.

- 3 mL whole blood at 20-25ºC or 2-8ºC.

Procedures for submission to central labs

- The specimen must be labeled appropriately, and standards must be adhered to. Lastly, ensure the requisition form is fully completed with all details.

- Label the tube with patient details and cross-check for accuracy with the requisition form.

- If refrigeration is needed, place the EDTA tube in a leak-proof container with cold packs and seal the packaging.

- Use an accredited courier or transport organization, keep to the appropriate temperatures during shipment, and follow the specimen's movement to the laboratory.

- Keep copies of the requisition form and tracking information, and document the chain of custody if applicable.

- Confirm receipt of the specimen by the central lab and follow up on the status of the test results through appropriate channels.

Procedures for microscopic examinations

- Blood smear preparation: Smear one drop of blood thinly on the slide, allowing it to air-dry quickly, not heat.

- Smear Staining Protocol: Wright Giemsa Stain the smear for direct visual examination of RBC, WBC, and platelet morphology.

- Microscopic Examination: Microscopically, at 100x oil immersion, examine RBC, WBC, and platelet morphology. Also, do WBC differential and abnormalities.

Documentation of Findings: Document and emphasize findings, for example, extreme anemia or leukemia, and alert the clinician quickly for life-threatening abnormalities.

Reagents or Media, Supplies, Equipment

| Procedure | Equipment | Reagents | Supplies |
| --- | --- | --- | --- |
| Hemoglobin | _ Spectrophotometer | _ Hemoglobin Reagent (Cyanmet Hemoglobin) _ Standard | _ Cuvettes / test tubes _ Sahli / micropipette |
| Hematocrit | _ Hematocrit Centrifuge _ Hematocrit Reader | _ none | _ Capillary Tube _ Sealer |
| Red Blood Cells | _ Pipette Shaker _ Tally Counter _ Microscope | _ RBC Diluent | _ RBC / micropipette _ Hemacytometer w/ Cover Slips _ WBC / micropipette _ Petri Dish |
| White Blood Cells |  | _ WBC Diluent |  |
| Platelet Count |  | _ Platelet Diluent |  |
| Complete Blood Count | _ Hematology Analyzer | _ Diluents _ Lyse _ Cleanse Solution _ Other Reagents | _ Test tubes _ Slides _ Staining Glasses |

Storage Requirements

- Materials or equipment should be capped and stored in a room-temperature (15-30°C) environment.

- Refrigerate reagents

Calibration

Equipment should be calibrated at the beginning of each shift. It should also be calibrated after any maintenance or service is done on the equipment.

Centrifuge:

- Clean the centrifuge

- Prepare the calibration standards necessary for the sample

- Check the temperature to the desired value

- Run the centrifuge for at least 30 minutes

- Verify the speed setting

- Check the balance of the rotor

- Make adjustments

- Document calibration

Hematology analyzers:

- Follow the manufacturer's instructions for calibration, which typically includes multiple runs of the calibrator material.

- Calculate the mean of the test runs.

- Compare the calculated mean to the assay target (i.e., the expected values for the calibrator material).

- If the results are within tolerance limits, no adjustments are necessary.

- If the results are close but require adjustment, accept the new calibration factors and adjust the instrument settings accordingly.

- Verify the calibration settings by testing known control samples, ensuring the analyzer provides accurate results.

Quality Control

Identify control materials to use

- Use commercially available hematology controls

Preparation, handling, and storage

- Follow the manufacturer's instructions and store according to the instructions

Frequency of testing

- Technicians should perform quality control testing every 8 hours, before the start of an operation or after any maintenance on the equipment.

Expected results

- Test results should fall within the expected range provided by the manufacturer.

Corrective actions

- If the results exceed the expected range, re-calibrate the equipment and redo the procedure

- Investigate the potential causes of error.

Recording and storage of QC data

- QC results must be documented on a logbook or digital system for ease of tracking and regulatory purposes.

Alternatives (If no QC materials are available)

- If QC materials are unavailable, use previous patents' results that are within the expected ranges

Step-by-step instructions

Quantitative Testing

- Collect blood samples into an EDTA tube

- Centrifuge the blood sample

- Analyze the sample on an automated hematology analyzer

Red Blood Cells

- Add the diluted blood to a hemocytometer chamber.

- Count the RBCs in the laid-out counting fields by using a microscope.

- Calculate the RBC count per microliter (µL)

White Blood Cells

- Dilute the blood into a hemocytometer chamber.

- Count the WBCs in the designated counting areas using a microscope.

- Determine the WBC count in microliter (µL)

Platelets

- Fill the undiluted blood into a hemocytometer chamber.

- The count of platelets in the defined areas under the microscope.

- Calculate the platelet count per µL:.

Haemoglobin

- Dilute the blood sample with Drabkin's reagent.

- Measure the absorbance of the diluted sample at 540 nm using a spectrophotometer.

- Calculate the hemoglobin concentration

Hematocrit

- Fill a capillary tube with blood, and seal both ends. Centrifuge the capillary tube at a set speed and time. Measure the length of the packed red cell column and the length of the blood column. Determine the hematocrit percentage

Qualitative Testing

- Prepare a blood smear.

- Stain the smear with Wright's stain.

- Examine under a microscope to assess cell morphology.

Interpretation

- Reference Ranges: Compare the quantitative results to established reference ranges for the patient's age, gender, and altitude.

- Morphological Abnormalities: Assess the peripheral blood smear for abnormal red blood cells, white blood cells, or platelet morphology.

- Correlation: Consider the correlation between CBC parameters and the patient's clinical history and symptoms.

- Differential Diagnosis: Based on the CBC results and clinical findings, suggest potential differential diagnoses.

Calculations

Mean Corpuscular Volume (MCV): * 10

Mean Corpuscular Hemoglobin (MCH): (Hgb / RBC) * 10

Mean Corpuscular Hemoglobin Concentration (MCHC): (Hgb/Hct) * 10

Reporting Results

Reference intervals:

| Blood Component | Abbreviation | Reference Range |
| --- | --- | --- |
| Red Blood Cells | WBC | Male: 4.35 trillion to 5.65 trillion cells/L Female: 3.92 trillion to 5.13 trillion cells/L |
| White Blood Cells | RBC | 3.4 billion to 9.6 billion cells/L |
| Hemoglobin | HGB | Male: 132 to 166 grams/L Female: 116 to 150 grams/L |
| Hematocrit | HT | Male: 38.3% to 48.6% Female: 35.5% to 44.9% |
| Platelet Count | Plt | Male: 135 billion to 317 billion/L Female: 157 billion to 371 billion/L |
| Mean Corpuscular Volume | MCV | 80-100 μm3 |
| Mean Corpuscular Hemoglobin | MCH | 25.4 - 34.6 pg/cell |
| MCH Concentration | MCHC | 31% - 36% Hb/cell |

Procedures for reporting abnormal results

- If results are not within the expected range, the technician should bring them to the physician’s attention. Healthcare providers may look at the results of other tests for comparison.

Reporting format

- Results are documented in a laboratory information system (LIS) or standardized form including patient details, test results, and interpretations.

Procedure Notes

Special precautions

- Avoid hemolysis.

- Label specimens correctly.

- Transport specimens promptly.

Possible sources of error

- Incorrect specimen handling

- Inadequate mixing

- Instrument malfunction

- Reagent deterioration

Answers to common problems

- Hemolyzed specimens: Recollect if possible.

- Counting errors: Double-check and verify.

- Reagent issues: Replace if necessary.

- Equipment problems: Repair or replace.

Limitations of Methods

This Method limits samples to human whole-blood

- Specimen rejection

- Reject clotted specimens and recollect.

- Interfering Substances and Conditions

Troubleshooting

If WBC, RBC, or Plt fails

- First, inspect and clean any blocked apertures. Then, prime the sweep flow several times to ensure fluid flow properly. If still not working, shut the system down as a last resort for further diagnostics.

If the Differential fails

- Flush the device to remove any debris blocking the flow cell that could interfere with the measurement

If Hgb fails

- Clean the Blood Sample Vessel to remove any residue from it and then prime the CBC module to ensure it is flowing normally and correctly, then do a hemoglobin blank verification test to validate if the machine is working correctly. Shut down the system if the problem is not apparent for troubleshooting.

References

Annex-A-General-Clinical-Laboratory. (2016). Health Facilities and Services Regulatory Bureau. Retrieved form https://caro.doh.gov.ph/wp-content/uploads /2017/04/Annex-A-General-Clinical-Laboratory.pdf

Calibration and Verification Process for Hematology Instruments. (n.d.). LabCE. Retrieved from https://www.labce.com/spg2870051_calibration_and_calibration_verification_process_

Complete Blood Count. (n.d.). Laboratory Alliance. Retrieved from https://www.laboratory alliance .com/tests/display/183

Dean L. Blood Groups and Red Cell Antigens [Internet]. Bethesda (MD): National Center for Biotechnology Information (US); 2005. Table 1, Complete blood count. Available from: https://www.ncbi.nlm.nih.gov/books/NBK2263/table/ch1.T1/

How to Calibrate Centrifuge: A Step-by-Stp Guide. (n.d.). Sciquip. Retrieved from https://sciquip.co.uk /news/how-to-calibrate-a-centrifuge-a-step-by-step-guide/

Mayo Clinic Staff. (2023). Complete Blood Count. Retrieved from https://www.mayoclinic.org/ tests-procedures/complete-blood-count/about/pac-20384919

Preanalytical Errors in a Hematology Laboratory: An Experience from a Tertiary Care Center. (2023). National Library of Medicine. Retrieved from https://www.ncbi.nlm.nih.gov/ pmc/articles/PMC9955620/#

The National Health and Nutrition Examination Survey Mobile Examination Center. (2019). Complete Blood Count Laboratory Procedure Manual. Retrieved from  https://wwwn.cdc.gov/nchs/data /nhanes/2017-2018/labmethods/CBC-J-MET-508.pdf

Effective Date

October 8, 2024

_____________________________

Signature of Laboratory Director