Complete Blood Count (CBC)

For Clinical Laboratory Section

PURPOSE

The purpose of this procedure is to provide a standardized method for the collection, processing, analysis, verification, and reporting of Complete Blood Count (CBC) results.

The CBC is used to evaluate the cellular components of peripheral blood and assist in the detection and monitoring of conditions such as anemia, infection, inflammation, and hematological disorders.

PRINCIPLE

The Complete Blood Count evaluates the quantity and characteristics of the major cellular components of blood, including red blood cells (RBCs), white blood cells (WBCs), hemoglobin (Hb), hematocrit (Hct), and platelets (PLT).

CBC analysis is primarily performed using an automated hematology analyzer. Depending on the instrument, cellular measurements are obtained through validated analytical methods such as electrical impedance, optical measurements, and spectrophotometric determination of hemoglobin.

When required, a peripheral blood smear may be prepared and examined microscopically to evaluate blood cell morphology and perform or verify a manual differential count.

SPECIMEN REQUIREMENTS

- Specimen Type

- Venous whole blood.

- Anticoagulant

- Ethylenediaminetetraacetic acid (EDTA).

- Collection Container

- Lavender-top EDTA tube.

- Required Volume

| Patient | Required Volume |
| --- | --- |
| Adult | 2 mL |
| Child | 0.5 mL |

- Specimen Handling

- Mix the specimen immediately after collection by gently inverting the tube 8–10 times.

- Do not vigorously shake the tube.

- Inspect the specimen for clots, hemolysis, inadequate volume, and other abnormalities.

- Analyze the specimen within 6 hours whenever possible.

- If analysis is delayed, store the specimen at 2–8°C for up to 24 hours, in accordance with laboratory policy.

- Before analysis, ensure that the specimen is adequately mixed.

REAGENTS AND MATERIALS

Reagents

- Hematology analyzer reagents

- Low-level hematology control

- Normal-level hematology control

- High-level hematology control

- Wright’s stain, when peripheral blood smear examination is required

Supplies

- Lavender-top EDTA tubes

- Clean glass microscope slides

- Disposable applicators/pipettes as applicable

- Biohazard waste containers

- Sharps container

- Laboratory tissue/wipes

Equipment

- Automated hematology analyzer

- Light microscope

- Computer/Laboratory Information System (LIS)

- Personal protective equipment (PPE)

CALIBRATION

- Ensure that the hematology analyzer is properly calibrated before patient testing.

- Perform calibration using the manufacturer’s recommended calibration materials and procedure.

- Follow the manufacturer’s specified calibration frequency.

- Verify that calibration results meet the established acceptance criteria.

- Record calibration activities and results in the laboratory maintenance/calibration log.

- If calibration is unacceptable, discontinue patient testing until the problem has been investigated and corrected.

QUALITY CONTROL

Procedure

- Prepare the low, normal, and high hematology control materials according to the manufacturer’s instructions.

- Run all required control levels at the beginning of each shift or according to the laboratory’s established QC schedule.

- Review the control results for all applicable CBC parameters.

- Compare the results with the laboratory’s established acceptable QC limits.

- Patient specimens may be analyzed only when the required QC results are acceptable.

Out-of-Control Results

If QC results are outside the acceptable range:

- Do not release patient results.

- Check the control material for proper storage, preparation, expiration, and handling.

- Check reagent levels and expiration dates.

- Review analyzer error messages and instrument status.

- Repeat QC when appropriate.

- Perform corrective action according to laboratory policy.

- Document the QC failure and corrective action.

- Resume patient testing only after acceptable QC has been established.

PROCEDURE

PRE-ANALYTICAL PHASE

A. Patient Identification

- Receive the laboratory request/requisition.

- Verify the patient’s identity using the required patient identifiers.

- Confirm that the patient information on the requisition corresponds with the specimen label.

- Confirm that a CBC has been requested.

B. Specimen Collection

- Perform hand hygiene and wear appropriate PPE.

- Prepare the required venipuncture materials and lavender-top EDTA tube.

- Perform venipuncture using standard aseptic technique.

- Collect the required volume of venous blood.

- Release the tourniquet according to standard phlebotomy procedure.

- Remove the collection device safely and dispose of sharps immediately in the designated sharps container.

- Gently invert the EDTA tube 8–10 times to ensure adequate mixing with the anticoagulant.

- Label the specimen immediately with the required patient identification and collection information.

C. Specimen Inspection

Inspect the specimen for:

- Proper identification

- Correct collection tube

- Adequate volume

- Proper mixing

- Clots

- Hemolysis

- Leakage or contamination

Specimens that do not meet laboratory acceptance criteria must be rejected or handled according to the established specimen-rejection policy.

ANALYTICAL PHASE

- Automated CBC Analysis

  - Verify that the hematology analyzer is operational.

  - Confirm that required calibration and QC procedures have been completed successfully.

  - Check reagent levels, waste-container status, and analyzer error messages.

  - Gently mix the EDTA specimen before analysis.

  - Enter or scan the patient’s specimen identification into the analyzer or LIS.

  - Select the appropriate CBC/whole-blood analysis mode.

  - Place the specimen in the designated sampling position.

  - Initiate the analysis according to the manufacturer’s operating instructions.

  - Allow the analyzer to aspirate and process the specimen.

  - Wait for the complete CBC result.

  - Review the numerical results and analyzer flags.

  - Investigate flagged or questionable results according to laboratory verification procedures.

- Peripheral Blood Smear / Manual Differential

  - A peripheral blood smear is performed when indicated by laboratory criteria or analyzer flags.

  - Mix the EDTA specimen gently.

  - Place a small drop of well-mixed blood near one end of a clean glass slide.

  - Position a second slide in front of the blood drop at an appropriate angle.

  - Allow the blood to spread along the edge of the spreader slide.

  - Push the spreader smoothly across the slide to produce a thin smear with a feathered edge.

  - Allow the smear to air-dry completely.

  - Stain the smear using Wright’s stain according to the laboratory’s validated staining procedure.

  - Examine the stained smear microscopically.

  - Evaluate RBC, WBC, and platelet morphology.

  - Perform a manual differential count when required.

  - Correlate microscopic findings with automated analyzer results.

  - Document significant morphological findings according to laboratory policy.

CBC PARAMETERS

The CBC may include the following parameters:

| Parameter | Description |
| --- | --- |
| WBC | White blood cell count |
| RBC | Red blood cell count |
| Hb | Hemoglobin concentration |
| Hct | Hematocrit |
| MCV | Mean corpuscular volume |
| MCH | Mean corpuscular hemoglobin |
| MCHC | Mean corpuscular hemoglobin concentration |
| RDW | Red cell distribution width |
| PLT | Platelet count |
| MPV | Mean platelet volume |

A differential count may include:

- Neutrophils

- Lymphocytes

- Monocytes

- Eosinophils

- Basophils

POST-ANALYTICAL PHASE

- Review the complete CBC results.

- Verify that QC was acceptable during the testing period.

- Review analyzer flags and available histograms/scattergrams.

- Investigate results that are inconsistent, questionable, or flagged.

- Perform repeat analysis or peripheral smear examination when required by laboratory policy.

- Correlate automated results with microscopic findings when applicable.

- Validate the final results.

- Enter or verify the results in the Laboratory Information System (LIS).

- Confirm that appropriate units and reference intervals are displayed.

- Report critical values immediately according to the laboratory’s critical-value notification procedure.

- Document all required corrective or follow-up actions.

REFERENCE RANGES

| Parameter | Reference Range |
| --- | --- |
| RBC | 4.2–6.1 × 10⁶/µL |
| WBC | 4,000–11,000/µL |
| Hb – Female | 12–16 g/dL |
| Hb – Male | 14–18 g/dL |
| Hct – Female | 37–47% |
| Hct – Male | 42–52% |
| Platelets | 150,000–400,000/µL |

Note: Reference intervals may vary according to the laboratory’s validated method, patient population, age, and sex. The laboratory’s established reference intervals shall be used for official reporting.

LIMITATIONS

- Hemolyzed specimens may produce inaccurate results.

- Clotted specimens may result in inaccurate cellular counts.

- Improper specimen collection, mixing, storage, or transportation may affect test results.

- Severe leukocytosis may interfere with certain CBC measurements.

- Lipemia may interfere with hemoglobin measurement.

- Improper analyzer calibration may affect analytical accuracy.

- Abnormal cell populations may cause analyzer flags and may require microscopic examination.

- Automated results should be reviewed according to established laboratory criteria before release.

TROUBLESHOOTING

- Analyzer Flags Inconsistent Results

Possible causes:

- Specimen abnormality

- Analyzer error

- Abnormal cell population

- QC or calibration issue

Corrective action:

- Review the analyzer flag and error message.

- Inspect the specimen for clots or other abnormalities.

- Verify QC results.

- Repeat analysis when appropriate.

- Perform additional testing or peripheral smear examination when indicated.

- If the problem persists, notify the authorized laboratory personnel and follow the manufacturer’s troubleshooting procedure.

2.  Clotted Blood Specimen

Possible cause: Inadequate mixing of blood with EDTA or improper collection.

Corrective action:

- Do not proceed with analysis if the specimen fails the laboratory’s specimen-acceptance criteria.

- Document the specimen rejection.

- Request recollection.

- Ensure that the replacement specimen is collected into the correct EDTA tube.

- Mix the new specimen gently by 8–10 inversions immediately after collection.

3.  Hemoglobin Interference Due to Lipemia

Corrective action:

- Review the analyzer result and flags.

- Confirm the presence of significant lipemic interference when applicable.

- Perform a plasma blank if supported by the laboratory’s validated procedure and analyzer methodology.

- Reanalyze or correct the result according to laboratory SOP.

- Document the corrective action.

SAFETY PRECAUTIONS

- Treat all blood specimens as potentially infectious.

- Wear appropriate PPE during specimen collection and processing.

- Avoid direct contact with blood and other biological materials.

- Dispose of needles and other sharps immediately in approved sharps containers.

- Do not recap used needles.

- Dispose of contaminated materials in appropriate biohazard containers.

- Clean and disinfect contaminated work surfaces according to laboratory policy.

- Perform hand hygiene after specimen handling and after removal of gloves.

- Follow institutional biosafety and infection-control procedures.

RESULT REPORTING

The final CBC report shall contain, as applicable:

- Patient identification

- Specimen identification

- Date and time of collection

- CBC results

- Appropriate units

- Laboratory reference intervals

- Analyzer flags or comments

- Manual smear findings, when applicable

- Critical-value notification documentation, when applicable

- Identification of the authorized laboratory personnel who verified/released the result

RECORD KEEPING

The following records shall be maintained according to laboratory policy:

- Patient CBC results

- Quality-control records

- Calibration records

- Analyzer maintenance records

- Corrective-action records

- Critical-value notification records

- Rejected specimen records